# Tab 1

Raspberry Pi Zero 2 W Drone Vision System with AI Detection capabilities

## 1. Objective

To design an embedded vision system for drones capable of performing real-time person detection using YOLOv8(or other light weight models) on a Raspberry Pi Zero 2 W(cheaper option)or Raspberry Pi4/5, and streaming annotated video (with bounding boxes) to a ground control system with minimal latency and extended range.

## 2. System Overview

The drone carries a Raspberry Pi Zero 2 W connected to a camera module and a 4G/5G modem (UNLIMITED RANGE as long as )
 The Pi performs on-board inference (YOLOv8-Nano) and overlays bounding boxes on detected persons.
 The processed video is then encoded (H.264) and transmitted to a ground system for display on VLC or a browser.

## 3. System Architecture

### 3.1 Drone-Side

| Component | Description | Function |
| --- | --- | --- |
| Raspberry Pi Zero 2 W | Quad-core Cortex-A53, 512 MB RAM | Edge AI inference + video streaming |
| Camera Module V2 (8 MP) | CSI-2 interface | Capture live video feed |
| 4G/5G USB Modem | Huawei E3372 | Long-range IP communication |
| Power Supply (5 V 2 A BEC) | From drone Li-Po battery | Stable regulated power |
| Lightweight Enclosure | 3D-printed | Protect electronics from vibration and fall damage |

Software Stack

- OS: Raspberry Pi OS Lite (64-bit)

- Frameworks: PyTorch Lite / Ultralytics YOLOv8

- Video: GStreamer (H.264 hardware encoding)

- Networking: WebRTC for low-latency streaming

### 3.2 Ground-Side (Receiver)

| Component | Description |
| --- | --- |
| Laptop / Ground Control Station | Receives and displays stream |
| VLC / FFplay / Browser (WebRTC) | Playback of annotated live feed |
| Optional: script for a Logger | Records detections and timestamps |

## 4. Power and Weight

| Component | Power (W) | Weight (g) |
| --- | --- | --- |
| Pi Zero 2 W | ~2 W | 13 g |
| Camera V2 | 0.5 W | 3 g |
| 4G Modem | 1–2 W | 25 g |
| Total | ~4.5 W | ~45 g |

## 5. References

- Raspberry Pi Zero 2 W Documentation – pi_docs

- Ultralytics YOLOv8 Docs – https://docs.ultralytics.com

- GStreamer SRT Streaming Guide – srt

# Tab 2

GSRT(GStreamer + SRT):

### GStreamer

- GStreamer is a multimedia framework that handles video/audio pipelines.

- It can take input from a camera, encode it, packetize it, and send it over a network using protocols like RTSP, RTP, or SRT.

### SRT (Secure Reliable Transport)

- A modern video streaming protocol (developed by Haivision).

- Works over UDP, but adds:

  - Error correction (retransmits missing packets),

  - Encryption, and

  - Adaptive latency control.

- It’s built specifically for low-latency video over unreliable networks (like 4G, Wi-Fi, or long-distance links).

#(udp speed+tcp like security)

## How It Works for the Drone

### On the Drone (Transmitter)

- Camera Feed (appsrc / v4l2src)

  - Captures raw frames from the Pi Camera.

- Convert to H.264

  - Uses the Pi’s GPU hardware encoder (v4l2h264enc) to compress frames efficiently.

- Pack into MPEG-TS

  - Converts raw H.264 frames into a streamable transport format (MPEG-TS).

- Send over SRT

  - The srtsink plugin pushes the video stream to the ground IP and port (for example, srt://192.168.0.10:9999).

### On the Ground (Receiver)

- SRT Socket Listening

  - VLC, FFplay, or GStreamer opens the same port (e.g., 9999) to receive packets.

- Automatic Resync

  - SRT handles network jitter, reorders packets, and ensures smooth video.

- Decode H.264 → Display

  - The player decodes and displays the video with low latency (100–200 ms typical).